9789.2  Article on both cables technical 1888

Western Mail (Perth, WA: 1885 - 1954), Saturday 14 July 1888, page 20

THE AUSTRALIAN CABLES.

(By W. J. Hancock M.S.T.E)

The Australian Duplicate Cable Line branches off from the Eastern Extension Telegraph Company's extensive system at Singapore. Messages by the old cable go from Singapore to Batavia; and from Batavia across Java to Banjoewangie, thence by a cable which skirts the lslands of Sumbawa , Flores, and Timor, and passes to the South of Samba, or Sandalwood Island.

The second cable goes from Singapore to Banjoewangie direct, and on again to Port Darwin. This cable goes to the north of all these islands, and lies in deep water.

The greatest distance between the two cables ma is off Sands Island where they are about 100 miles apart.

The 1st cable was laid, by the Eastern Extension Telegraph Co., unsubsidised in 1871, and before the overland Telegraph Line to Port Darwin was finished; the messages being conveyed by horses, express over the unfinished portion until the 21st of October 1872, when through telegraphic communication was first established.

The advantage of rapid communication with the rest of the world was found to be so great that when interruptions occasionally occurred, involving loss and inconvenience, the question of duplicating the cable were seriously considered, the result being that a second cable was laid between Singapore and Port Darwin in 1870, by the same company, the colonies of Victoria, South Australia, New South Wales, and Western Australia jointly  paying, in proportion to their respective populations, an annual subsidy of £34,400 for a period of 20 years.

The conductor of the 1871 cable consists of seven stranded copper wires of about No, 20 B.W. gauge each: surrounded with several layers of gutta percha, forming the insulation.

This is again enclosed in an armour consisting of iron wires, to protect and give the necessary strength to the cable. This is finally wound with hemp and asphalt. The shore side of the cable are more heavily protected by very much thicker wires, The main cable is about 1in. in. diameter, and weighs 3 ½ tons to the mile, while the shore ends weighs about 11 ½ tons per mile.

The 1879 cable, is very similar, except that an intermediate size of cable is used, as well as the main and shore cable. The shore end of this cable weighs about 10 tons, intermediate 3½, and main 1½ tons per mile. In this cable between the gutta purcha core and the cable between the gutta purcha core and the iron wires brass tape is wound as a protection against the attacks of the teredo nevalis, a little worm that bores trough the gutta purcha, thereby destroying the insulation. These cables cost roughly about £100 per mile.

The system adopted for working Is not the same as is used for ordinary telegraph lines, in which direct currents are transmitted_ The submarine cable is analogous to an immense and attenuated Leydon jar, or condenser, the conductor being the inner, the sheath and water the outer coating, and the gutta percha acting as the glass insulator. Thus the cable, so to speak, holds a charge of electricity, and by varying the amount and polarity of the charge, according to a  pre concerted arrangements of signals, the messages can be signaled.

The instrument generally used on a long cable is Sir W. Thomson’s Syphon Recorder, which consists of a very fine, wire, surrounded between the poles of two powerful electro-magnets. Through this coil of wire the currents received travel.

Fixed to this coil is a very fine glass syphon tube which conducts the ink from the reservoir to a strip of paper drawn past the end of the glass tube at a uniform speed.  Thus, this glass pen records every movement of the coil to right or left. The suspended coil and tube will be attracted to one magnet or the other according to the polarity and strength of the current received. Thus, it will be easily understood how a code of signals can be made to represent the alphabet, as in the International Code, letters are represented by movements of the coil and glass tube, which are recorded on the paper strip. Thus, A is represented by one movement to the right and one to the left; B one to the right and three to the left; C one right one left, one right, one left and so on.

The speed of the instrument on the newest cables is about 18 words of 5 letters each per minute, but when worked in duplex the rate is about doubled. Cable working is slow compared with normal telegraph, which varies from 25 to 40 words when worked by hand, to from 200 to 1000 per minute in various rapid automatic machines.

When a cable is broken one way of finding the distance to the break is approximately as follows :- The resistance which the entire length of the conductor of the cable offers to the passage of the electric current is known, and tests are usually taken each morning before work. The unit of resistance is called the " Ohm".  Thus, the resistance of the 1879 cable is about 18,000 ohms, or about 11 1/2 ohms per mile, in good condition. When the (cable is broken and the inside conductor exposed, the current has not got to travel the entire length, hut only as for as the break, and back by the iron sheathing and water, the resistance of which is taken as nil.

The resistance is much less, and from the testing instrument the number of ohms can be found, and as so many miles go to 1 ohm the number of mile distance of the fault is

calculated. The matter is; however, not so simple as various circumstances occur which interfere with the result. For instance, a small portion of the conductor may be exposed, a local current can set up between the copper and iron wires in the sea water, polarisation, and other troubles.

To fish up the cable, a grapnel and stout wire rope is used, the steamer going, slowly across the line of the cable until it is caught, when it is hauled on board, examined, and repaired.

The course of telegraphic messages from Perth to London is as follows

|  | Miles of land line | Miles of Cable | Total |
| --- | --- | --- | --- |
| Perth - Albany | 261 |  | 261 |
| Albany - Eucla | 762 |  | 762 |
| Eucla-Port Augusta | 759 |  | 759 |
| Port Augusta - Alice Springs | 1036 |  | 1036 |
| Alice Springs - Port Darwin | 898 |  | 898 |
| Port Darwin - Banjoewangie |  | 1150 | 1150 |
| Banjoewangie - Batavia | 480 |  | 480 |
| Batavia - Singapore |  | 553 | 553 |
| Singapore - Penang |  | 399 | 399 |
| Penang – Madras |  | 1280 | 1280 |
| Madras - Bombay | 650 |  | 650 |
| Bombay - Aden |  | 1662 | 1662 |
| Aden - Suez |  | 1346 | 1346 |
| Suez - Alexandria | 224 |  | 224 |
| Alexandria - Malta |  | 828 | 828 |
| Malta - Gibraltar |  | 1008 | 1008 |
| Gibraltar - Falmouth |  | 1061 | 1061 |
| Falmouth - London | 350 |  | 350 |
|  | 5,410 | 9,287 | 14,697 |

In the event of interruption of the cable connections at certain points of the regulated route, messages can he sent by way of Penang or Rangeon, or via Saigon, Bankok and Tavoy (Burma), or by the Indo - European Company's system, through Russia and across Europe, or by the Great Northern Companies system, across Siberia.

The last route is as follows.

|  | Miles |
| --- | --- |
| Singapore to Hong Kong | 1876 |
| Hong Kong to Shanghai | 1265 |
| Shanghai to Posiett Bay Siberia | 1380 |
| Across Siberia and Russia to lieban Baltic | 6394 |
| Lieban to new Boggin, England | 1656 |
| Total | 12,571 |

The distance of the proposed route across Canada is approximately

| Sydney to Vancouver B. Colombia | 8900 |
| --- | --- |
| Across Canada | 3400 |
| Canada to Ireland | 2450 |
| Total | 14800 |

Messes. Millar Bros proposed cable from the North West Coast of Australia would be

| N.W. coast of Australia to Madras | 3500 |
| --- | --- |
| Madras to London | 8409 |
|  | 11909 |

This would have the advantage of avoiding the volcanic regions through which both of the current cables run.

The break in communications may be caused by some of those violent volcanic disturbances which occur so frequently in the neighbouring Malay Archipelago.

According to the latest test the distance of the break is about 858 miles from Port Darwin on the old line and on the new cable 898 miles. The Commander of the Myrndon reports that Mount Tomboro is in active eruption. This troublesome volcano is on the north side of Sumbawa Island. To the south of this island lie both the cables, not very far apart. This point is, however, about 900 miles from Port Darwin, and about 200 miles from Ranjoewangie. That the old cable was previously defective and the new cable suddenly from some causes failed, thus cutting off all communications, is probably the most reasonable explanation.

If the cables are only broke in one place, a steamer would not take long to pick them up and repair the damage. But if much of the cable is destroyed by violent disturbance over a considerable area ( similar to what occurred in 1882 at Krakatoa) new pieces will have to be put in, and unless the cable steamers (one of which was reported to have been a Shanghai and the other at New Zealand) on their return have sufficient length to repair the damage, or unless a piece of the old cable is cut out, and spliced into the new one, or vis versa, some months may elapse before through telegraph communication will be restored.

But it is utterly impossible to foretell how long the break may last, as it depends on the damage, the cause of which we have no means of knowing at present, shut out as we are from the rest of the world.

The present cost of cablegrams from Perth to London are as follows:-

Public messages via Europe direct 9s 4d per word of 10 letters;

Cochin China 12s 2d;

Hong Kong 15s 1d

During the year 1886, 46,667 messages to and from Australia passed through the Port Darwin cables representing £256,527, or an average per message of £5 10s